# Vehicle Features — Functional Guide

BLE App + IoT Tracker · Observable Behaviour Specification

Ahraz Shamim · Technical Product Manager, Vecmocon Technologies

# Document control

| Field | Value |
| --- | --- |
| Document | Vehicle Features — Functional Guide |
| Audience | Customers and integrators using the BLE app and IoT tracker |
| Scope | Describes observable behaviour, not implementation |
| Platform | EC200U IoT tracker · companion MCU · BLE mobile app |
| Author | Ahraz Shamim, Technical Product Manager |

Note on scope. This document specifies what the system does as seen from outside — what a user, integrator or support engineer can observe. Implementation detail lives in the internal project context document.

# 1. System at a glance

The IoT tracker is the centre of the system. It is powered by the auxiliary battery and is always awake. The main battery is a separate unit that identifies itself over CAN.

Bluetooth and the motion sensor never switch off. They are how the vehicle is woken and how it is protected, so every power-saving behaviour in this document is designed around that constraint rather than overriding it.

## Signal map

| From | To | Carries |
| --- | --- | --- |
| BLE App | IoT Tracker | Lock / Unlock / Boot / Ignition commands |
| IoT Tracker | Companion MCU | Relayed commands (boot release) |
| Companion MCU | IoT Tracker | Wheel-sensor voltage |
| Main Battery | IoT Tracker | CAN 0x540 heartbeat |
| Motion Sensor | IoT Tracker | Movement and tamper events |
| IoT Tracker | Cloud | Telemetry, when online |
| IoT Tracker | Nearby Apple devices | Find My beacon → Find My network |
| IoT Tracker | Outputs | Buzzer · Wheel lock · Ignition relay |

# 2. Power modes

Three modes, chosen automatically to save power on a parked vehicle.

| Mode | What works | What is off | Typical use |
| --- | --- | --- | --- |
| Active | Everything — tracking, cloud, CAN, app | — | Riding / in use |
| Derated | App, motion sensing, battery link | GPS, cloud / mobile data | Vehicle stirred but not in use |
| Low Power | App, motion sensing | GPS, cloud / mobile data, battery link | Parked and undisturbed |

Power draw drops roughly 4× in Low Power compared with a fully connected device.

## Transitions

| From | To | Trigger |
| --- | --- | --- |
| Active | Low Power | Ignition OFF for 2 minutes |
| Low Power | Derated | Small movement detected |
| Derated | Low Power | No movement for 2 minutes |
| Low Power | Active | App command or significant motion |
| Derated | Active | App command or theft detected |

## What the user notices

- The app always connects, in every mode.

- In Derated and Low Power the vehicle stops appearing online; live location is stale.

- Sending any app command returns the vehicle to Active. Reconnecting to the network takes approximately 20–40 seconds, so the first cloud update after waking is not instant.

- Recovery is immediate — the device never sleeps, so there is no boot delay.

# 3. BLE app commands

| Command | Effect | Refused when |
| --- | --- | --- |
| Lock | Short beep; anti-theft armed | Ignition is ON |
| Unlock / Ignition-off | If ignition ON, turns it off; otherwise double beep and disarms anti-theft | Never |
| Boot unlock | Opens the boot (relayed to the companion MCU) | Companion MCU unreachable |
| Ignition ON | Energises the ignition relay | Main battery not detected, or vehicle is locked |

Every command returns an acknowledgement to the app — ACK … on success or NAK … with the reason on refusal — so a refusal is always visible rather than silent.

Buzzer and the ignition circuit. The buzzer is powered from the ignition circuit, so the device briefly raises ignition to make lock and unlock beeps audible. This is not an ignition-on event: the vehicle stays logically off.

# 4. Main-battery detection and ignition control

The main battery announces itself with a repeating CAN heartbeat. Detection is asymmetric on purpose — quick to trust, slow to distrust.

| Condition | Result |
| --- | --- |
| Heartbeat received | PRESENT — immediately, one heartbeat is enough |
| Silent for 10 seconds | ABSENT for power logic |
| Silent for 30 seconds | ABSENT for ignition |

## Consequences

- Present is instant. One heartbeat is enough.

- Absent needs sustained silence, so a single dropped CAN message never causes a false “battery removed”.

- Ignition cannot be switched on without a live main battery.

- If the battery is removed while riding, ignition is switched off automatically after 30 seconds. No app command is needed.

- If ignition is already off when the battery goes absent, ignition stays off and any new Ignition-ON request is refused.

# 5. Anti-theft

Armed by Lock, disarmed by Unlock. Fully automatic once armed — no app interaction required.

## Detectors

Three independent detectors. Any one of them triggers the alarm.

| Detector | Detects |
| --- | --- |
| Wheel-sensor movement | Vehicle being wheeled or rolled |
| Significant motion | Vehicle lifted or carried |
| Small nudge | Tamper attempt on a stationary vehicle |

Each detector has a debounce, so a one-off bump or vibration does not raise an alarm.

## Alarm behaviour

- The alarm runs in 10-second cycles and repeats for as long as tampering continues.

- While active it drives the buzzer, the wheel lock and the ignition line.

- Unlock always wins — it stops the alarm immediately.

- Detection keeps working even if the motion sensor fails, because the wheel sensor is a separate channel.

- Anti-theft is active in every power mode, including Low Power.

# 6. Behaviour after a power cut or reset

On reset, outputs start in a safe state with everything off. What the device then restores differs per item, because the risk differs per item.

| Item | Survives reset? | Why |
| --- | --- | --- |
| Locked / unlocked state | Yes | A reset must not silently unlock a parked vehicle |
| Ignition on/off | No — always off | Safety: a relay must only energise on a fresh, deliberate command |
| Power mode | No — starts Active | Safest default; re-enters Low Power normally |

So a vehicle locked before a power interruption is still locked and still guarded afterwards, and anti-theft re-arms automatically. A rider whose vehicle resets must re-send Ignition ON.

# 7. Find My — locating the vehicle without mobile network

The tracker continuously broadcasts an Apple “Find My” (OpenHaystack) beacon over Bluetooth. Any passing iPhone or iPad silently relays the sighting to Apple’s Find My network, where only the owner can decrypt the location.

## Recovery path

| Step | Actor | Action |
| --- | --- | --- |
| 1 | Vehicle (no SIM / no coverage) | Broadcasts BLE beacon |
| 2 | Passer-by’s iPhone | Relays sighting silently |
| 3 | Apple Find My network | Stores encrypted report |
| 4 | Owner | Decrypts — the only party who can |

## Why it matters

- Works with no SIM, no mobile coverage and no cloud connection. It needs only a nearby Apple device.

- Runs in every power mode, including Low Power, because Bluetooth never switches off.

- It is a last-resort locator: if a thief removes the main battery, or the vehicle is in a basement with no signal, the beacon keeps advertising on the auxiliary battery.

## How it coexists with the app

The device advertises the Find My beacon and keeps its normal VEC-<IMEI> name and connectable services at the same time — so the BLE app still finds, connects to and controls the vehicle exactly as before. The Bluetooth address is derived from the device’s Find My key, so each vehicle appears as its own distinct tag.

Provisioning requirement. Each unit needs its own unique Find My key. Devices sharing one key are indistinguishable in the Find My network and cannot be located individually. This is a manufacturing-line requirement, not a firmware one.

# 8. How it all works together

Bluetooth, motion sensing and anti-theft (when locked) are always available. Power mode controls only GPS, mobile data, cloud and the main-battery link.

| Interaction | Rule |
| --- | --- |
| Anti-theft vs. power saving | Anti-theft outranks power saving. An alarm can occur in any mode, and temporarily drives the ignition line to sound the buzzer. |
| Ignition vs. power mode | Ignition depends on the main-battery link, which is switched off in Low Power. The app command wakes the device first so the link is restored — but the vehicle must be awake for ignition to be permitted. |
| Lock vs. power mode | Independent. Locking does not change the power mode, and the power mode does not disarm anti-theft. |
| Motion as shared trigger | A small nudge lifts Low Power to Derated; significant motion returns the device to Active and can raise an alarm if the vehicle is locked. |

## Design principles behind these rules

- The device never truly sleeps. It sheds radios instead. This costs a little standby current versus a true sleep state and buys zero boot delay on recovery, plus anti-theft that is live in Low Power.

- Asymmetric thresholds where the two errors have different costs. Battery present versus absent, false alarm versus missed tamper, lock state versus ignition state — each is timed to the cost of getting it wrong in that direction.

- No silent failures. Every refused command names its reason; every degraded mode is visible to the user rather than hidden behind a spinner.